## `Key Features

- 10W power amplifier, 
Frequency range: 1.7 – 2.2 GHz
Automatic level control (ALC)

- 0.9dB noise figure LNA
Frequency range: 1.5 – 2.7 GHz
Custom frequency band at no extra charge.

- Can be configured as half-duplex (with tx/rx switch included) or full-duplex (diplexer not included)

- Single 26-30V DC power supply.
Includes 5V 3A power supply output to power other modules (such as transceiver and modem).

- Transmit gain: > 40 dB
Receive gain: > 35 dB

- Temperature and transmit power reported as analog signals and digital measurements.

- Half-duplex Tx/Rx switch time < 2µs

- RF interfaces: 
50 Ohm SMA connectors (antenna side)
50 Ohm UMCC connectors (transceiver side)

- Monitoring and Control through asynchronous serial (LVTTL) or USB.

## Applications Example

### 3-module satellite modem

For the latest data sheet, please refer to the ComBlock web site: comblock.com/com4103.html.

These specifications are subject to change without notice.

For an up-to-date list of ComBlock modules, please refer to comblock.com/product_list.html

## Block Diagram

Half-Duplex configuration (-A option)

Full-Duplex configuration (-B option)

## Electrical Interface

| RF |  | Definition |
| --- | --- | --- |
| TX_IN |  | Power amplifier input. (J7) UMCC connector, 50Ohm. AC coupled. -12 dBm max operational. +20 dBm max no damage. |
| Antenna port / TX_OUT/ANT |  | Antenna port (when configured as half-duplex), or power amplifier out (when configured as full duplex) Maximum no damage input level: + 20 dBm (J8) SMA right-angle female connector, 50Ohm. |
| RX_IN |  | LNA input (when configured as full-duplex) Maximum no damage input level: + 20 dBm (J6) SMA right-angle female connector, 50Ohm. |
| RX_OUT |  | LNA output (J3) UMCC connector, 50Ohm. |
| Controls Flat flexible cable |  | Definition |
| RX_TXN |  | Half-duplex receive/transmit# control signal. ‘0’ = transmit ‘1’ = receive. Ignored when configured for full duplex. In half-duplex, the transmit output is muted when RX_TXN = ‘1’. LVTTL input. Recommended guard time: 5us. Flat flexible cable connector pin 8 |
| TX_EN |  | Binary transmit section enable. Active high. ‘1’ = to power the transmit section ‘0’ = to save power in a receive-only configuration. Unlike the fast RX_TXN switch signal, this signal cannot be switched dynamically because of the power supply slow rise time. LVTTL input. Flat flexible cable connector pin 7 |
| RFOUT_ENB |  | Binary transmit section enable. Active low. ‘0’ = to enable the transmit RF path ‘1’ = to mute the power amplifier. This control signal is fast. LVTTL input. Flat flexible cable connector pin 6 |
| TX_GAIN_CTRL_EXT |  | Analog 0 – 3.3V Controls the transmitter gain when enabled. Flat flexible cable connector pin 4 |
| Status Flat flexible cable |  | Definition |
| Transmitted power |  | Analog monitoring signals Flat flexible cable connector pin 2 Range 0 – 2.5V |
| Board temperature |  | Analog monitoring signals Flat flexible cable connector pin 3 Range 0 – 2.5V |
| Monitoring & Control |  |  |
| USB | Mini-USB connector Type AB Full speed / Low Speed |  |
| Async. Serial | LVTTL-level (NOT RS232!) asynchronous serial, 115.2 Kbaud. Through the flat flexible cable and card edge. |  |
| Power |  |  |
| Power supply input | 26-30VDC; Solder pads. Power consumption is 1.2A typ. |  |
| 5VDC output | 5VDC to power other modules. 3A maximum output. Solder pads. |  |

### Absolute Maximum Ratings

| Supply voltage | -0.3V min, +36V max |
| --- | --- |
| Control signals | -0.3V min, +5.5V max |
| RF inputs | +20 dBm |

## Configuration

An entire ComBlock assembly comprising several ComBlock modules can be monitored and controlled centrally over a single connection with a host computer. Connection types include built-in types:

- USB

- Asynchronous serial (LVTTL levels)

or connections via adjacent ComBlocks.

The module configuration is stored in non-volatile memory.

### Configuration (Basic)

The easiest way to configure the COM-4103 is to use the ComBlock Control Center software supplied with the module on CD. In the ComBlock Control Center window detect the ComBlock module(s) by clicking the  Detect button, next click to highlight the COM-4103 module to be configured and click the  Settings button to display the Basic Settings window shown below.

### Configuration (Advanced)

Alternatively, users can access the full set of configuration features by specifying 8-bit control registers as listed below. These control registers can be set manually through the ComBlock Control Center or by software using the ComBlock API (see www.comblock.com/download/M&C_reference.pdf)

The module configuration parameters are stored in non-volatile memory. All control registers are read/write.

| Parameters | Configuration |
| --- | --- |
| LNA enable/disable | Enable(1) / Disable (0) LNA. The objective is to reduce power consumption when the receive path is unused. REG0(0) |
| Power amplifier enable/disable | Enable(1) / Disable(0) the power amplifier. The objective is to reduce power consumption when the transmit path is unused. As the command switches the PA_+28V power supply on/off, it is relatively slow. REG0(1) |
| Fast power amplifier enable/disable | Enable(1) / Disable(0) the power amplifier at the gate. Controls the power amplifier gates. Switching is fast, but the biasing remains enabled. REG0(2) |
| RX/TX switch | Controls the RX/TX switch position when the board is configured for half-duplex. 0 = receive path open 1 = transmit path open In the full duplex option –B, this control signal is ignored. The rx/tx switch opens the transmit path when the power amplifier is turned on and switches the transmit path off otherwise. REG0(3) |

| Flat flexible cable signals enabled/disabled | The above commands can be overruled by signals received from an external module over a flat flexible cable (J4 connector). Enable(1) or disable(0) REG0(7) |
| --- | --- |
| Transmitter ALC loop | 0 = open loop. Transmitter gain is fixed by control registers below. 1 = internal ALC loop. This feature is disabled when the board is configured for external transmit gain control. REG1(0) |
| Fixed transmitter gain | Transmitter gain when ALC is disabled (open loop transmit gain). 10-bit: 0 for the minimum gain, 1023 for the maximum gain. The transmitter gain change is enacted upon writing to REG3. REG2: bits 7:0 (LSB) REG3(1:0): bits 9:8 |
| ALC target transmitted power | Expressed in the same units as the 12-bit value from the transmit power detector. (see Monitoring section). Any change in ALC target transmitted power is enacted upon writing to REG5. REG4: bits 7:0 (LSB) REG5(3:0): bits 11:8 |
| Analog test point selection | Select which analog signal is to be monitored at the TP1 test point 0= disabled 1 = temperature sensor 2 = transmit power sensor REG6(1:0) |

## Monitoring

The monitoring panel opens by clicking on the  Settings button and the “Monitoring” tab. Alternatively hexadecimal values are shown by clicking on the  Status button.

| Parameters | Monitoring |
| --- | --- |
| Power supply check | SREG0(0): power good1 +3.3V |
| Board temperature | Thermistor output SREG1 = LSB SREG2(3:0) = MSB |
| RF output power | Power detection between the power amplifier and the tx/rx switch. See TX_POWER_DET in schematic. SREG3 = LSB SREG4(3:0) = MSB |

### Test Points

Test points are provided for easy access by an oscilloscope probe.

| Test Point | Definition |
| --- | --- |
| TP1 | Analog test point (temperature sensor or transmit power sensor as selected by the user) |
| Red LED | Red when one of these conditions occur: - 3.3V supply voltage too low |

## Operations

### Heat Sink

One side of the board consists of a ground plane without any component or exposed non-ground vias. It is designed to be tightly connected to a heat radiating surface such as a metal enclosure or the supplied heat sink. Four mounting holes sized for 
4-40 screws are located (0.160,0.160) from the four corners of the 3”x3” board.

Heat sink dimension: 3” x 3.5” x 1.05”

The transmitter should never be enabled if the heat sink is not assembled tightly on the ground side of the board.

Use of thermal grease (thermal compound) between the board and the heat sink is recommended for optimized heat transfer.

### Controls

This module can be controlled through USB (using the user-friendly ComBlock control center GUI or API) or through a 10-signal flat flexible cable.

To insert the FFC, pull out the black section of the zero-insertion force connector J4, insert the FFC end with the contacts down and lock the connector by pushing the black section back. Be sure to properly align the FFC with the connector.

### Internal vs external transmit gain control

The transmit path gain can be controlled either internally under software control or externally via the TX_GAIN_CTRL_EXT signal on pin 8 of the flat flexible cable connector.

The selection of internal versus external is done by soldering one of two surface-mount resistors:

- Internal control: R54 = 0R, R62 = open

- External control: R54 = open, R62 = 0R

Internal software control is configured by default. 
Please select internal/external transmit gain control at the time of ordering.

## Schematics

The schematics are available on the ComBlock CD shipped with every module (in the “Hardware schematics” folder).

## Performance

### Transmit power

The transmit power vs frequency is plotted below.

Reference: 9.2 dBm = 10W

### Transmitted modulated signal

Input: 1 MHz QPSK signal, +2 dBm

Output (after 30.8 dB attenuation): +41.2 dBm

Input: 1 MHz QPSK signal, -7 dBm

Output (after 30.8 dB attenuation): +35.4 dBm

### ALC

An automatic Level Control (ALC) can be enabled for the transmitter. The ALC dynamic range is approximately 20 dB, so it is important to position the input level and target output level so that the ALC does not reach its min/max boundaries. The ALC response time is captured below for input steps of 10 dB (horizontal time scal is 2.5s/division)

### Receiver gain (full duplex –B option)

100 MHz: 46 dB

500 MHz 49 dB

1 GHz: 45 dB

1.5 GHz: 40 dB

2 GHz: 37 dB

2.5 GHz: 37 dB

3 GHz: 35 dB

## Mechanical Interface

## Pinout

### Mini USB Connector, J1

The COM-4103 is a USB device with a mini type AB connector.  (G = GND)

### Flat Flexible Cable Connector, J4

In addition to USB, the COM-4103 can also be monitored and controlled through a flat flexible cable connected to the J4 ZIF board-mount connector, 10 positions, 1mm pitch, right angle.

Compatible flat flexible cables:

1mm pitch, 0.3mm thickness, 10 positions.

Parlex USA  100R50-152B

Molex Inc  0210390255

Wurth Electronics Inc 686610152001

### I/O Compatibility List

(not an exhaustive list)

I/O

COM-3506 [400MHz - 3GHz] Transceiver

COM-4410 [70 MHz – 2.2 GHz] 4-channel quadrature RF modulators

COM-400x quadrature modulators

## Accessories

### Included

- USB cable

- UMCC to SMA stubs (2)

### Recommended

- Thermal grease for better contact between board and heat sink

- Flat flexible cable

## ComBlock Ordering Information

COM-4103-A  
L/S-band 10W Power Amplifier / LNA, half-duplex

COM-4103-B  
L/S-band 10W Power Amplifier / LNA, full-duplex
(diplexer not included)

At the time of ordering, please select:

- Internal/external transmit gain control

- Receive frequency band (for LNA filtering)

MSS  •  18221-A Flower Hill Way  • 
Gaithersburg, Maryland 20879 •  U.S.A.

Telephone: (240) 631-1111    
Facsimile: (240) 631-1676    
E-mail: sales@comblock.com

Ultra-Miniature Coaxial Connector to SMA adapter is supplied.