## Overview

The COM-1509SOFT is a high-speed compact Viterbi error correction decoder written in VHDL. Parametrized parallel implementation allows the decoding of Gbps encoded inputs even on low-cost FPGAs such as the Xilinx Spartan-6.

The entire VHDL source code is deliverable.

Key features and performance:

- Serial or parallel implementation for either minimum size or maximum speed.

- Flexible dynamic (i.e. at runtime) user-selected configuration:

  - Hard (1-bit) or soft(4-bit) decision decoding

  - G0,G1,G2 generator polynomials

  - Constraint length K=3,5,7,9

  - Puncturing patterns for rates 2/3,3/4,5/6,7/8

  - 2 or 3 parity bits

- Support for erasures (code puncturing)

- Reduced latency: K*25 bits

- Self-synchronizing to automatically align the incoming encoded stream with the periodic encoding/puncturing pattern

- Out-of-sync indicator to synchronize external demodulator (for removing PSK demod phase ambiguity for example)

- Provided with IP core:

  - VHDL source code

  - Matlab .m file to generate stimulus files for VHDL simulation at various signal to noise ratios

  - VHDL testbench

## Target Hardware

The code is written in generic standard VHDL so that it can be ported to a variety of FPGAs. The code is developed and tested on a Xilinx Spartan-6 FPGA.

It can be easily ported to most Xilinx and other FPGAs as the only used FPGA primitive (a 16Kbit block RAM) is encapsulated in a generic wrapper.

## Overall Block Diagrams

Demodulator connectivity

## Configuration

Synthesis-time configuration parameters

The following constants are user-defined in the  COM1509pkg.vhd component prior to synthesis. These parameters generally define the size of the decoder embodiement.

| Synthesis-time configuration parameters |  |
| --- | --- |
| Parameters | Configuration |
| Number of parallel decoders instantiated NDEC | Typical values: 1 (for minimum size serial architecture), 2,4,8 (for maximum throughput parallel architecture) |
| Hard/Soft-decision decoding HARD_SOFTN_DECISION | ‘1’ for 1-bit width input ’0’ for 4-bit width input |
| Maximum constraint length K_MAX | Valid values are 3,5,7,9 |
| Maximum number of parity bits N_PARITY_BITS_MAX | Valid range 2 to 7 |

Run-time configuration parameters

The user can set and modify the following controls at run-time through the top level component interface:

| Transmitter |  |
| --- | --- |
| Parameters | Configuration |
| Generator polynomials G | Traditionally expressed as a K-bit octal number. For example CCSDS generator polynomials are G(0) = o”171’ and G(1) = o”133” G(6:2) unused |
| Constraint length K | Valid values are 3,5,7,9 Must be less than or equal to K_MAX |
| Convolutional encoding rate R1/R2 | Valid values are 1/2, 1/3, 2/3, 3/4, 5/6, 7/8 |
| DIFF | Differential decoding enabled(1)/disabled(0) |

### Codes

Convolutional codes are defined by the generator polynomials Gx and puncturing pattern if any. Codes defined in various standards are listed below.

0 marks an erasure during puncturing.

| CCSDS |  |
| --- | --- |
| Constraint length | K=7 |
| Generator Polynomials | G0 = o”171” G1 = o”133” |
| Puncturing pattern rate 1/2 | G0; 1. G1; 1. |
| Puncturing pattern rate 2/3 | G0; 1 0. G1; 1 1. |
| Puncturing pattern rate ¾ | G0; 1 0 1. G1; 1 1 0. |
| Puncturing pattern rate 5/6 | G0; 1 0 1 0 1. G1; 1 1 0 1 0. |
| Puncturing pattern rate 7/8 | G0; 1 0 0 0 1 0 1. G1; 1 1 1 1 0 1 0. |

| Intelsat |  |
| --- | --- |
| Constraint length | K=7 |
| Generator Polynomials | G0 = o”133” G1 = o”171” |
| Puncturing pattern rate 1/2 | G0; 1. G1; 1. |
| Puncturing pattern rate 2/3 | G0; 1 1. G1; 1 0. |
| Puncturing pattern rate ¾ | G0; 1 1 0. G1; 1 0 1. |
| Puncturing pattern rate 5/6 | G0; 1 1 0 1 0. G1; 1 0 1 0 1. |
| Puncturing pattern rate 7/8 | G0; 1 1 1 1 0 1 0. G1; 1 0 0 0 1 0 1. |

WiMAX

### Hard/Soft-decision decoding

### Differential Decoding

Differential decoding can be used following FEC decoding as specified in Intelsat IESS-308/309. This feature can be enabled/disabled by software.

This mode compensates for any bit inversion occurring in the transmission channel (for example at a BPSK demodulator which cannot resolve the inherent 180deg phase ambiguity). This ambiguity resolution mechanism has a cost: the bit error rate is doubled.

## I/Os

CLK: input

The synchronous clock.  The user must provide a global clock (use BUFG). The CLK timing period must be constrained in the .ucf file associated with the project.

SYNC_RESET: input

Synchronous reset. The reset MUST be exercised at least once to initialize the internal variables.

DATAIN(31:0):

32-bit input packing eight soft-quantized samples. Input samples are soft-quantized with 4-bits. Format is offset binary whereby 0000 represents a ‘strong’ 0, 1111 a strong ‘1’, 0111 the weakest ‘0’ and 1000 the weakest ‘1’.

When using hard-decision decoding, encoded bits are placed at location 31,27,…7,3 in the vector. Other locations are ignored.

Packing order for the eight samples is from the most significant side to the least significant side of this array.

DATAIN_VALID:

1 CLK-wide pulse indicating that DATAIN is valid.

DATAIN_READY:

A flow-control signal indicating whether the decoder is ready to accept new DATAIN input samples.

--// CONFIGURATION

-- The configuration parameters below can be changed dynamically at run-time.

-- Use SYNC_RESET after such a configuration change.

G: in slvGENPOLYxN_PARITY_BITS_MAX;

-- G0,G1,etc. generator polynomials.

-- For each generator polynomial, unused generator polynominal taps at indices (K_MAX:K+1) (if K < K_MAX) must be set to '0'.

N_PARITY_BITS: in integer range 2 to 7;

-- actual number of parity bits (i.e. generator polynomials).

-- valid range: 2 - 7

-- MUST be less than or equal to the hardware capabilities set by N_PARITY_BITS_MAX defined in com1509pkg.vhd

K: in integer range 3 to 9;

-- constraint length.

-- valid values: 3,5,7,9

-- MUST be less than or equal to the hardware capabilities set by K_MAX defined in com1509pkg.vhd

R1: in integer;

R2: in integer;

-- Convolutional encoding rate R1/R2

-- Valid rates: 1/2, 1/3, 1/7, 2/3, 3/4, 5/6, 7/8

-- DECODER OUTPUTS

-- 2 output types: 1-bit serial or 8-bit parallel.

DATAOUT1b: out std_logic;

DATAOUT1b_VALID: out std_logic;

-- 1-bit serial

DATAOUT8b: out std_logic_vector(7 downto 0);

DATAOUT8b_VALID: out std_logic;

-- 8-bit parallel, order: MSb first.

--// MONITORING (when BER_MEASUREMENT_EN = '1')

-- BER_WINDOW_LENGTH and BER_THRESHOLD are defined in com1509pkg

-- Note: at very low SNRs, spurious OUT_OF_SYNC and unreliable LOCK_STATUS may occur.

-- In this case, it is recommended to either confirm the OUT_OF_SYNC condition over several BER measurement windows, or

-- to increase the BER_WINDOW_LENGTH.

BER: out std_logic_vector(31 downto 0);

-- encoded stream bit error rate

BER_VALID: out std_logic;

-- read BER at rising edge of CLK when BER_VALID = '1'

OUT_OF_SYNC: out std_logic;

-- 1 CLK-wide pulse to indicate that the input coding groups are not aligned with the

-- transmitted coding groups, i.e. received bit in the G0 bin not actually generated by G0, etc.

-- This information is typically used by external circuitry to realign the received encoded

-- bit stream with the expected parity groups or puncturing pattern, or to correct for

-- PSK demodulator phase ambiguities, etc.

LOCK_STATUS: out std_logic

-- a continuous '1' when measured BER is below the set threshold.

-- toggling when unlocked

## Monitoring

### Bit Error Rate Measurement

The decoder estimates the bit error rate on the encoded bit stream by comparing the actual received bit stream with an estimate of the transmitted bit stream. This estimate is generated by re-encoding the nearly error-free decoded bit stream.

The algorithm is based on the proposition that the decoded bit stream is nearly error-free. If the decoded bit stream were error-free, then the re-encoded bit stream would be the actual transmitted encoded bit stream before bit errors occur in the transmission channel.

The bit error rate is computed over a window of 1000 bits. This measurement window length BER_WINDOW_LENGTH is a constant set in com1509pkg.vhd.

### Self Synchronization

When the decoder detects that the bit error is greater than a set threshold, it attempts to re-synchronize. Different thresholds are used for the various encoding rates. See the BER_THRESHOLD_xx constants  in com1509pkg.vhd.

Digital status registers are read-only.

| Viterbi FEC decoder monitoring |  |
| --- | --- |
| Parameters | Monitoring |
| Synchronized | Synchronized SREG15(0) |
| Decoder built-in BER | The Viterbi decoder computes the BER on the received (encoded) data stream irrespective of the transmitted bit stream. Encoded stream bit errors detected over the specified BER measurement window. SREG16 = bits 7 – 0 (LSB) SREG17 = bits 15 – 8 SREG18 = bits 23 – 16 (MSB) |

Note: multi-words status registers such as frequency offset or BER, are latched upon reading status register SREG10.

## Troubleshooting checklist

- Place modem in loopback mode (REG35 = 0x07) while sending a PRBS-11 test sequence (REG5 = 0x03). Be sure to direct the demodulated bit stream to the BER measurement (REG36 = 0x03).
 
Check the status registers for

  - Demodulator is locked: 
SREG14 = 0x01

  - Viterbi decoder synchronized: SREG15 = 0x01

  - No Viterbi decoder errors: 
SREG16/17/18 = 0

  - BER measurement is synchronized:
SREG27 = 0x01

  - No BER errors:
SREG23/24/25/26 = 0

  - Bits are being received at the HDLC decoder output:
SREG19/20/21/22 counter keeps increasing at a rate consistent with the modulation rate.

## I/Os

### 1-bit synchronous serial

The user should always check the “Clear-To-Send”  CTS_OUT flag before sending additional data bits to the modulator.

## Implementation Block Diagram

Transmitter

## Software Licensing

The COM-1509SOFT is supplied under the following key licensing terms:

- A nonexclusive, nontransferable license to use the VHDL source code internally, and

- An unlimited, royalty-free, nonexclusive transferable license to make and use products incorporating the licensed materials, solely in bitstream format, on a worldwide basis.

The complete VHDL/IP Software License Agreement can be downloaded from http://www.comblock.com/download/softwarelicense.pdf

## Configuration Management

The current software revision is 1.

| Directory | Contents |
| --- | --- |
| /doc | Specifications, user manual, implementation documents |
| /src | .vhd source code, .ucf constraint files, .pkg packages. One component per file. |
| /sim | Test benches, Matlab .m files to generate encoded input files with various Eb/N0 and to compute expected BER. |
| /bin | .ngc, .bit, .mcs configuration files |

Key files:

Xilinx ISE project file: com-1509_ISE131.xise

.ucf constraint file example when used on the COM-1500 FPGA platform:
\src\COM1509.ucf

## VHDL development environment

The VHDL software was developed using the following development environment:

- Xilinx ISE 13.4 with XST as synthesis tool

- Xilinx ISE Isim as VHDL simulation tool

The entire project fits within a Xilinx Spartan-6 LX45. Therefore, the ISE project can be processed using the free Xilinx WebPack tooks.

### Device Utilization Summary

The implementation size depends essentially on three key user-defined parameters in com1509pkg.vhd, namely:

- Hard/Soft-decision decoding HARD_SOFTN_DECISION

- Maximum constraint length K_MAX
(the actual constraint length K is dynamically configurable at run-time but cannot exceed the hardware capabilities defined by K_MAX)

- Maximum number of parity bits N_PARITY_BITS_MAX
(the actual number of parity bits N_PARITY_BITS is dynamically configurable at run-time but cannot exceed the hardware capabilities defined by N_PARITY_BITS_MAX)

Device: Xilinx Spartan-6

|  | Used 1 decoder | % of Spartan-6 LX45 |
| --- | --- | --- |
| Registers | 2960 | 5% |
| LUTs | 4610 | 16% |
| Block RAM/FIFO | 3 | 2% |
| DSP48A1s | 0 | 0% |
| GCLKs | 1 | 6% |
| DCMs | 0 | 0% |
| PLL_ADVs | 0 | 0% |

### Clock and decoding speed

The entire design uses a single global clock CLK.

Typical maximum clock frequencies for various FPGA families are listed below:

| Device family | fCLK | Max encoded/decoded bit rate 1 decoder 2 parity bits |
| --- | --- | --- |
| Xilinx Kintex 7 -2 | 344 MHz | 344/172MHz |
| Xilinx Virtex-6 -2 | 272 MHz | 272/136 Mbits/s |
| Xilinx Spartan-6 –3 | 166 MHz | 166/83 Mbits/s |

The maximum decoded bit rate is

fCLK *NDEC / N_PARITY_BITS

Where NDEC is the number of parallel decoders embodied in the design.

## Ready-to-use Hardware

The COM-1509SOFT was developed on, and therefore ready to use on the following commercial off-the-shelf hardware platform:

FPGA development platform

COM-1500 FPGA + DDR2 SODIMM socket + ARM development platform

## Xilinx-specific code

The VHDL source code is written in generic VHDL with only one Xilinx primitive (encapsulated in a generic wrapper for easy substitution). No Xilinx CORE is used. The Xilinx primitives are:

- RAM block

## VHDL components overview

### Top level

VITERBI_DECODER.vhd is the decoder top component in this hierarchical design. It’s role is to segment the continuous input stream of encoded (hard or soft-quantized) bits into fixed-length blocks which can be processed by NDEC parallel decoders. The decoded bit stream is then reassembled into a seamless output stream. This component also adjusts the alignment of the input encoded bit stream with the coding and puncturing periodic pattern. Indeed, the follow-on Viterbi algorithm (VA.vhd) can only work properly if a parity bit encoded by generator polynomial Gx in the transmitter is decoded with the same generator polynomial at the receiver.

DATA_SPLITTER.vhd implements the continous input stream segmentation.

COM1509.vhd: is mostly a use example when the Viterbi decoder is implemented on a ComBlock COM-1500 FPGA development platform. This component includes a Viterbi decoder, clock generation, interface to a supervisory microcontroller (8-bit address/data bus to exchange control registers REG and status registers SREG). CLK_P is the main processing clock.

BER3.vhd synchronizes with the received bit stream and counts the number of bit error when a PRBS-11 sequence is being transmitted.

INFILE2SIM.vhd reads an input file. This component is used by the testbench to read a soft-quantized or hard-quantized encoded bit stream generated by the viterbi_dec_gen_input.m Matlab program for various Eb/No cases.

SIM2OUTFILE.vhd writes three 12-bit data variables to a tab delimited file which can be subsequently read by Matlab (load command) for plotting or analysis.

## Decoder simulation

The internal digital signal processing can be illustrated by the following screenshots obtained through VHDL simulation. A Matlab program (siggen_psk.m) generates a stimulus file which mimics A/D converter samples at the receiver input.

Internal FPGA signals can be saved into Matlab loadable (load command) text files by setting the constant SIMULATION to ‘1’ in various VHDL components.

State metrics

## Reference documents

[1]

## ComBlock Ordering Information

COM-1509SOFT  Gbps VITERBI DECODER, VHDL SOURCE CODE / IP CORE

## Contact Information

MSS  •  18221-A Flower Hill Way  • 
Gaithersburg, Maryland 20879 •  U.S.A.

Telephone: (240) 631-1111   
Facsimile: (240) 631-1676    
E-mail: info@comblock.com