This 7-module ComBlock assembly creates a full-duplex modem, including error correction, modulation and TCP-IP interface.

The baseline configuration is as follows:

1 Msymbols/s QPSK modulation (2 Mbits/s)

915 MHz RF center frequency

Use of external 10 MHz clock to minimize frequency error between transmitter and receiver.

Turbo Product Code, frame length 4096.

TCP-IP socket at address 172.16.1.128, port 1024.

HDLC encoding.

Configuration:

a) please be sure to load the COM-5003 option C software into personality 4. To verify which option is running, highlight the COM-5003, click on Settings (3rd Button from the left), click on the “Advanced” button. The current option shown at the bottom of the panel should be C. Revision 5 or greater.

Check list:

- transmit RF frequency synthesizer locked onto external 10 MHz frequency reference: test point SYNC_LOCK should be constantly low.

- receive RF frequency synthesizer locked onto external 10 MHz frequency reference: test point PLL_LOC should be constantly low.

- AGC working properly: using an oscilloscope, check the COM-3001 test points TP1 and TP2. These baseband waveforms should be 1Vpp.

- Demodulator lock: the COM-1001 test point 1 should be high.

- Frame synchronization: the COM-7001 TP3 should show periodic pulses spaced every 2.06ms and a few extra pulses. TP4 should be the same without the extra spurious pulses. Please be sure to enable the ‘peak detect’ function on a digital oscilloscope as the pulses are extremely narrow and would not be shown otherwise.

- The COM-7001 TP1 and TP2 should show periodic pulses spaced 2.06ms. There should be no missing frame.

- Using a hyperterminal, connect to 172.16.1.128 port 1024. 
For each character typed, there should be an echo. 
In addition, for each character typed in the hyperterminal window, there the COM-5003 should show increments in the two byte counters (status register 1 and 5).